Running Both SISEAO Systems Simultaneously

Overview

You can run both the full Firebase-connected rpi_appliance_controller.py and the lite local version at the same time. This gives you:

- Primary System: Full Firebase integration with cloud sync

- Backup System: Local-only control when Firebase is down

- Dual Access: Web interface + Firebase cloud access

Architecture

┌─────────────────────────────────────────┐

│           Raspberry Pi                   │

├─────────────────────────────────────────┤

│  rpi_appliance_controller.py            │

│  ├── Firebase Integration               │

│  ├── GPIO Control (Primary)             │

│  ├── Pattern Detection                  │

│  └── Cloud Synchronization              │

├─────────────────────────────────────────┤

│  siseao_lite_backup.py                  │

│  ├── Local Web Interface (Port 5001)    │

│  ├── GPIO Monitoring (Read-only)        │

│  ├── Local Energy Tracking              │

│  └── Backup Control (Emergency)         │

└─────────────────────────────────────────┘

Setup Instructions

1. Modify the Lite Version for Dual Operation

# /home/pi/siseao_lite/siseao_lite_backup.py

import sqlite3

import json

import time

import threading

from datetime import datetime, timedelta

from flask import Flask, jsonify, render_template_string

import RPi.GPIO as GPIO

# Configuration

DB_PATH = '/home/pi/siseao_lite/siseao_backup.db'

WEB_PORT = 5001  # Different port to avoid conflict

MONITOR_INTERVAL = 5  # Check GPIO states every 5 seconds

# Device GPIO Configuration - MUST MATCH rpi_appliance_controller.py

DEVICES = {

"001": {"gpio": 18, "wattage": 60, "name": "Living Room Light"},

"002": {"gpio": 19, "wattage": 100, "name": "Kitchen Light"},

"003": {"gpio": 20, "wattage": 150, "name": "Bedroom Fan"},

"004": {"gpio": 21, "wattage": 200, "name": "Air Conditioner"}

}

# GPIO setup for monitoring only (not controlling)

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

class SISEAOBackup:

def __init__(self):

self.primary_controller_active = True

self.last_gpio_states = {}

self.setup_database()

self.setup_gpio_monitoring()

self.start_monitoring()

def setup_database(self):

"""Create SQLite database for backup system"""

conn = sqlite3.connect(DB_PATH)

cursor = conn.cursor()

cursor.execute('''

CREATE TABLE IF NOT EXISTS device_states (

device_id TEXT PRIMARY KEY,

device_name TEXT,

wattage REAL,

gpio_pin INTEGER,

current_status TEXT DEFAULT 'OFF',

last_change TIMESTAMP DEFAULT CURRENT_TIMESTAMP,

controlled_by TEXT DEFAULT 'primary'

)

''')

cursor.execute('''

CREATE TABLE IF NOT EXISTS state_changes (

id INTEGER PRIMARY KEY AUTOINCREMENT,

device_id TEXT,

timestamp TIMESTAMP DEFAULT CURRENT_TIMESTAMP,

old_status TEXT,

new_status TEXT,

source TEXT,

duration_minutes REAL DEFAULT 0,

energy_wh REAL DEFAULT 0

)

''')

cursor.execute('''

CREATE TABLE IF NOT EXISTS system_status (

id INTEGER PRIMARY KEY,

primary_controller_active BOOLEAN DEFAULT 1,

backup_mode_active BOOLEAN DEFAULT 0,

last_check TIMESTAMP DEFAULT CURRENT_TIMESTAMP

)

''')

# Initialize devices

for device_id, config in DEVICES.items():

cursor.execute('''

INSERT OR IGNORE INTO device_states (device_id, device_name, wattage, gpio_pin)

VALUES (?, ?, ?, ?)

''', (device_id, config['name'], config['wattage'], config['gpio']))

# Initialize system status

cursor.execute('''

INSERT OR REPLACE INTO system_status (id, primary_controller_active, backup_mode_active)

VALUES (1, 1, 0)

''')

conn.commit()

conn.close()

print("✅ Backup database initialized")

def setup_gpio_monitoring(self):

"""Setup GPIO pins for monitoring (input mode)"""

for device_id, config in DEVICES.items():

try:

# Set as input with pull-down to read current state

GPIO.setup(config["gpio"], GPIO.IN, pull_up_down=GPIO.PUD_DOWN)

# Read initial state

initial_state = "ON" if GPIO.input(config["gpio"]) == GPIO.HIGH else "OFF"

self.last_gpio_states[device_id] = initial_state

print(f"📡 Monitoring GPIO {config['gpio']} ({config['name']}): {initial_state}")

except Exception as e:

print(f"❌ Error setting up GPIO {config['gpio']}: {e}")

def check_primary_controller(self):

"""Check if primary controller is active by monitoring a status file or process"""

try:

# Method 1: Check if primary controller process is running

import psutil

for proc in psutil.process_iter(['pid', 'name', 'cmdline']):

try:

cmdline = ' '.join(proc.info['cmdline'] or [])

if 'rpi_appliance_controller.py' in cmdline:

return True

except:

pass

return False

except ImportError:

# Method 2: Check for a heartbeat file

heartbeat_file = '/tmp/siseao_primary_heartbeat'

try:

with open(heartbeat_file, 'r') as f:

last_heartbeat = float(f.read().strip())

return (time.time() - last_heartbeat) < 30  # 30 second timeout

except:

return False

def monitor_gpio_states(self):

"""Monitor GPIO states and log changes"""

conn = sqlite3.connect(DB_PATH)

cursor = conn.cursor()

changes_detected = False

for device_id, config in DEVICES.items():

try:

current_gpio_state = "ON" if GPIO.input(config["gpio"]) == GPIO.HIGH else "OFF"

last_known_state = self.last_gpio_states.get(device_id, "OFF")

if current_gpio_state != last_known_state:

changes_detected = True

# Calculate energy usage if device was ON

duration_minutes = 0

energy_wh = 0

if last_known_state == "ON":

# Get last change time

cursor.execute('''

SELECT timestamp FROM state_changes

WHERE device_id = ? ORDER BY timestamp DESC LIMIT 1

''', (device_id,))

result = cursor.fetchone()

if result:

last_change = datetime.fromisoformat(result[0])

duration_minutes = (datetime.now() - last_change).total_seconds() / 60

energy_wh = (config['wattage'] * duration_minutes) / 60

# Log the change

cursor.execute('''

INSERT INTO state_changes

(device_id, old_status, new_status, source, duration_minutes, energy_wh)

VALUES (?, ?, ?, ?, ?, ?)

''', (device_id, last_known_state, current_gpio_state,

'primary_controller', duration_minutes, energy_wh))

# Update current state

cursor.execute('''

UPDATE device_states

SET current_status = ?, last_change = CURRENT_TIMESTAMP,

controlled_by = ?

WHERE device_id = ?

''', (current_gpio_state,

'backup' if not self.primary_controller_active else 'primary',

device_id))

self.last_gpio_states[device_id] = current_gpio_state

print(f"📊 {config['name']}: {last_known_state} → {current_gpio_state}")

if energy_wh > 0:

print(f"⚡ Energy: {energy_wh:.2f} Wh ({duration_minutes:.1f} min)")

except Exception as e:

print(f"❌ Error monitoring {device_id}: {e}")

# Update system status

self.primary_controller_active = self.check_primary_controller()

cursor.execute('''

UPDATE system_status

SET primary_controller_active = ?,

backup_mode_active = ?,

last_check = CURRENT_TIMESTAMP

WHERE id = 1

''', (self.primary_controller_active, not self.primary_controller_active))

conn.commit()

conn.close()

return changes_detected

def emergency_control_device(self, device_id, action):

"""Emergency device control when primary controller is down"""

if self.primary_controller_active:

return {

"success": False,

"error": "Primary controller is active. Use primary system for control."

}

if device_id not in DEVICES:

return {"success": False, "error": "Device not found"}

config = DEVICES[device_id]

# Get current status

conn = sqlite3.connect(DB_PATH)

cursor = conn.cursor()

cursor.execute("SELECT current_status FROM device_states WHERE device_id = ?", (device_id,))

result = cursor.fetchone()

current_status = result[0] if result else "OFF"

# Determine new status

if action == "toggle":

new_status = "OFF" if current_status == "ON" else "ON"

elif action in ["on", "ON", "turn-on"]:

new_status = "ON"

elif action in ["off", "OFF", "turn-off"]:

new_status = "OFF"

else:

conn.close()

return {"success": False, "error": "Invalid action"}

try:

# Switch GPIO to output mode temporarily

GPIO.setup(config["gpio"], GPIO.OUT)

GPIO.output(config["gpio"], GPIO.HIGH if new_status == "ON" else GPIO.LOW)

# Log the change

cursor.execute('''

INSERT INTO state_changes

(device_id, old_status, new_status, source, duration_minutes, energy_wh)

VALUES (?, ?, ?, 'backup_emergency', 0, 0)

''', (device_id, current_status, new_status))

# Update state

cursor.execute('''

UPDATE device_states

SET current_status = ?, last_change = CURRENT_TIMESTAMP, controlled_by = 'backup'

WHERE device_id = ?

''', (new_status, device_id))

conn.commit()

conn.close()

# Switch back to input mode for monitoring

time.sleep(0.1)

GPIO.setup(config["gpio"], GPIO.IN, pull_up_down=GPIO.PUD_DOWN)

self.last_gpio_states[device_id] = new_status

print(f"🚨 EMERGENCY CONTROL - {config['name']}: {current_status} → {new_status}")

return {

"success": True,

"device_id": device_id,

"status": new_status,

"mode": "emergency_backup",

"warning": "Primary controller is offline"

}

except Exception as e:

conn.close()

return {"success": False, "error": f"GPIO control failed: {e}"}

def start_monitoring(self):

"""Start background monitoring thread"""

def monitor_loop():

while True:

try:

self.monitor_gpio_states()

time.sleep(MONITOR_INTERVAL)

except Exception as e:

print(f"❌ Monitor error: {e}")

time.sleep(MONITOR_INTERVAL)

monitoring_thread = threading.Thread(target=monitor_loop, daemon=True)

monitoring_thread.start()

print("🔍 GPIO monitoring started")

def get_all_devices(self):

"""Get all device states"""

conn = sqlite3.connect(DB_PATH)

cursor = conn.cursor()

cursor.execute('''

SELECT device_id, device_name, wattage, current_status, last_change, controlled_by

FROM device_states ORDER BY device_name

''')

devices = cursor.fetchall()

conn.close()

device_list = []

for device in devices:

device_list.append({

"device_id": device[0],

"device_name": device[1],

"wattage": device[2],

"status": device[3],

"last_change": device[4],

"controlled_by": device[5],

"can_control": not self.primary_controller_active

})

return device_list

def get_energy_summary(self, days=7):

"""Get energy usage summary"""

conn = sqlite3.connect(DB_PATH)

cursor = conn.cursor()

# Energy usage by device

cursor.execute('''

SELECT s.device_id, s.device_name, SUM(c.energy_wh) as total_wh, COUNT(*) as cycles

FROM state_changes c

JOIN device_states s ON c.device_id = s.device_id

WHERE c.timestamp >= datetime('now', '-{} days')

AND c.energy_wh > 0

GROUP BY s.device_id, s.device_name

ORDER BY total_wh DESC

'''.format(days))

usage_by_device = cursor.fetchall()

# System status

cursor.execute('SELECT * FROM system_status WHERE id = 1')

system_status = cursor.fetchone()

# Currently ON devices

cursor.execute("SELECT device_id, device_name, wattage FROM device_states WHERE current_status = 'ON'")

devices_on = cursor.fetchall()

conn.close()

return {

"total_energy_wh": round(sum([row[2] for row in usage_by_device]), 2),

"current_power_w": sum([row[2] for row in devices_on]),

"devices_on": len(devices_on),

"total_devices": len(DEVICES),

"primary_controller_active": system_status[1] if system_status else False,

"backup_mode_active": system_status[2] if system_status else True,

"usage_by_device": [

{"device": row[1], "energy_wh": round(row[2], 2), "cycles": row[3]}

for row in usage_by_device

]

}

# Initialize backup controller

backup_controller = SISEAOBackup()

# Flask Web App

app = Flask(__name__)

# Enhanced HTML template for backup system

HTML_TEMPLATE = '''

<!DOCTYPE html>

<html>

<head>

<title>SISEAO Backup Controller</title>

<meta name="viewport" content="width=device-width, initial-scale=1">

<style>

* { box-sizing: border-box; margin: 0; padding: 0; }

body { font-family: 'Segoe UI', sans-serif; background: #f0f2f5; padding: 10px; }

.container { max-width: 1000px; margin: 0 auto; }

.header { background: linear-gradient(135deg, #ff6b6b 0%, #ee5a24 100%);

color: white; padding: 20px; border-radius: 10px; margin-bottom: 20px; text-align: center; }

.status-bar { padding: 15px; margin-bottom: 20px; border-radius: 5px; text-align: center; font-weight: bold; }

.status-primary { background-color: #d4edda; border: 1px solid #c3e6cb; color: #155724; }

.status-backup { background-color: #fff3cd; border: 1px solid #ffeaa7; color: #856404; }

.status-emergency { background-color: #f8d7da; border: 1px solid #f5c6cb; color: #721c24; }

.stats { display: grid; grid-template-columns: repeat(auto-fit, minmax(200px, 1fr));

gap: 15px; margin-bottom: 20px; }

.stat-card { background: white; padding: 20px; border-radius: 10px; box-shadow: 0 2px 10px rgba(0,0,0,0.1); text-align: center; }

.stat-value { font-size: 2em; font-weight: bold; color: #ff6b6b; }

.stat-label { color: #666; margin-top: 5px; }

.devices { display: grid; grid-template-columns: repeat(auto-fit, minmax(300px, 1fr)); gap: 15px; }

.device { background: white; border-radius: 10px; padding: 20px; box-shadow: 0 2px 10px rgba(0,0,0,0.1); }

.device.on { border-left: 5px solid #4CAF50; }

.device.off { border-left: 5px solid #f44336; }

.device-header { display: flex; justify-content: space-between; align-items: center; margin-bottom: 15px; }

.device-name { font-size: 1.1em; font-weight: bold; }

.device-status { padding: 5px 15px; border-radius: 20px; color: white; font-weight: bold; }

.status-on { background: #4CAF50; }

.status-off { background: #f44336; }

.device-info { color: #666; margin-bottom: 15px; }

.controls { display: flex; gap: 10px; }

.btn { padding: 10px 20px; border: none; border-radius: 5px; cursor: pointer;

font-weight: bold; transition: all 0.3s; }

.btn:hover:not(:disabled) { transform: translateY(-2px); }

.btn:disabled { opacity: 0.5; cursor: not-allowed; }

.btn-emergency { background: #ff6b6b; color: white; }

.btn-refresh { background: #FF9800; color: white; padding: 15px 30px;

font-size: 1.1em; margin: 20px auto; display: block; }

.controller-badge { font-size: 12px; padding: 3px 8px; border-radius: 12px;

background: #e9ecef; color: #495057; }

.controller-primary { background: #d4edda; color: #155724; }

.controller-backup { background: #fff3cd; color: #856404; }

.warning { background: #fff3cd; border: 1px solid #ffeaa7; color: #856404;

padding: 15px; border-radius: 5px; margin-bottom: 20px; }

</style>

</head>

<body>

<div class="container">

<div class="header">

<h1>🛡️ SISEAO Backup Controller</h1>

<p>Monitoring & Emergency Control System</p>

</div>

<div id="statusBar" class="status-bar">

Loading system status...

</div>

<div id="warningBox" class="warning" style="display: none;">

⚠️ Primary controller is offline. Emergency control is available.

</div>

<div class="stats">

<div class="stat-card">

<div class="stat-value" id="totalDevices">-</div>

<div class="stat-label">Total Devices</div>

</div>

<div class="stat-card">

<div class="stat-value" id="devicesOn">-</div>

<div class="stat-label">Currently ON</div>

</div>

<div class="stat-card">

<div class="stat-value" id="currentPower">-</div>

<div class="stat-label">Current Power (W)</div>

</div>

<div class="stat-card">

<div class="stat-value" id="weeklyEnergy">-</div>

<div class="stat-label">Weekly Energy (Wh)</div>

</div>

</div>

<div id="devices" class="devices">

<div style="text-align: center; padding: 50px; color: #666;">Loading devices...</div>

</div>

<button onclick="refreshAll()" class="btn btn-refresh">🔄 Refresh All</button>

</div>

<script>

let systemData = {};

async function emergencyControl(deviceId, action) {

if (systemData.primary_controller_active) {

alert('Primary controller is active. Use the main system for control.');

return;

}

if (!confirm(`Emergency control: ${action.toUpperCase()} device ${deviceId}?`)) {

return;

}

try {

const response = await fetch(`/emergency/${deviceId}/${action}`, {method: 'POST'});

const result = await response.json();

if (result.success) {

showNotification(`Emergency: ${result.status}`, 'warning');

setTimeout(refreshAll, 1000);

} else {

showNotification(`Error: ${result.error}`, 'error');

}

} catch (error) {

showNotification('Connection error', 'error');

}

}

async function loadDevices() {

try {

const response = await fetch('/devices');

const devices = await response.json();

const container = document.getElementById('devices');

container.innerHTML = '';

devices.forEach(device => {

const isOn = device.status === 'ON';

const canControl = device.can_control;

const deviceClass = isOn ? 'device on' : 'device off';

const statusClass = isOn ? 'device-status status-on' : 'device-status status-off';

const controllerClass = device.controlled_by === 'primary' ? 'controller-primary' : 'controller-backup';

container.innerHTML += `

<div class="${deviceClass}">

<div class="device-header">

<div class="device-name">${device.device_name}</div>

<div class="${statusClass}">${device.status}</div>

</div>

<div class="device-info">

💡 ${device.wattage}W | ID: ${device.device_id}<br>

<span class="controller-badge ${controllerClass}">

${device.controlled_by === 'primary' ? '🌐 Primary' : '🛡️ Backup'}

</span>

</div>

<div class="controls">

<button onclick="emergencyControl('${device.device_id}', 'on')"

class="btn btn-emergency" ${!canControl ? 'disabled' : ''}>

Emergency ON

</button>

<button onclick="emergencyControl('${device.device_id}', 'off')"

class="btn btn-emergency" ${!canControl ? 'disabled' : ''}>

Emergency OFF

</button>

</div>

</div>

`;

});

} catch (error) {

document.getElementById('devices').innerHTML =

'<div style="text-align: center; padding: 50px; color: #666;">❌ Failed to load devices</div>';

}

}

async function loadSystemData() {

try {

const response = await fetch('/energy');

systemData = await response.json();

// Update stats

document.getElementById('totalDevices').textContent = systemData.total_devices;

document.getElementById('devicesOn').textContent = systemData.devices_on;

document.getElementById('currentPower').textContent = systemData.current_power_w;

document.getElementById('weeklyEnergy').textContent = systemData.total_energy_wh;

// Update status bar

const statusBar = document.getElementById('statusBar');

const warningBox = document.getElementById('warningBox');

if (systemData.primary_controller_active) {

statusBar.className = 'status-bar status-primary';

statusBar.innerHTML = '🌐 PRIMARY ACTIVE - Monitoring Mode';

warningBox.style.display = 'none';

} else {

statusBar.className = 'status-bar status-emergency';

statusBar.innerHTML = '🚨 BACKUP MODE - Emergency Control Available';

warningBox.style.display = 'block';

}

} catch (error) {

console.error('Failed to load system data:', error);

}

}

function showNotification(message, type) {

const notification = document.createElement('div');

notification.textContent = message;

notification.style.cssText = `

position: fixed; top: 20px; right: 20px; z-index: 1000;

padding: 15px 20px; border-radius: 5px; color: white; font-weight: bold;

background: ${type === 'success' ? '#4CAF50' : type === 'warning' ? '#FF9800' : '#f44336'};

transform: translateX(400px); transition: transform 0.3s;

`;

document.body.appendChild(notification);

setTimeout(() => notification.style.transform = 'translateX(0)', 100);

setTimeout(() => {

notification.style.transform = 'translateX(400px)';

setTimeout(() => document.body.removeChild(notification), 300);

}, 4000);

}

function refreshAll() {

loadDevices();

loadSystemData();

}

// Auto-refresh every 8 seconds

setInterval(refreshAll, 8000);

// Initial load

refreshAll();

</script>

</body>

</html>

'''

@app.route('/')

def index():

return HTML_TEMPLATE

@app.route('/devices')

def get_devices():

return jsonify(backup_controller.get_all_devices())

@app.route('/emergency/<device_id>/<action>', methods=['POST'])

def emergency_control(device_id, action):

return jsonify(backup_controller.emergency_control_device(device_id, action))

@app.route('/energy')

def get_energy():

return jsonify(backup_controller.get_energy_summary())

@app.route('/status')

def status():

return jsonify({

"system": "SISEAO Backup Controller",

"timestamp": datetime.now().isoformat(),

"primary_active": backup_controller.primary_controller_active,

"monitoring_mode": True

})

if __name__ == '__main__':

print("🛡️ Starting SISEAO Backup Controller...")

print(f"🌐 Web interface: http://localhost:{WEB_PORT}")

print(f"📱 Mobile access: http://[PI_IP]:{WEB_PORT}")

print("📡 Monitoring primary controller...")

try:

app.run(host='0.0.0.0', port=WEB_PORT, debug=False)

except KeyboardInterrupt:

print("\n👋 Stopping SISEAO Backup Controller...")

GPIO.cleanup()

2. Modify Primary Controller to Create Heartbeat

Add this to your existing rpi_appliance_controller.py:

# Add this function to rpi_appliance_controller.py

def create_heartbeat():

"""Create heartbeat file for backup system"""

def heartbeat_loop():

while True:

try:

with open('/tmp/siseao_primary_heartbeat', 'w') as f:

f.write(str(time.time()))

time.sleep(5)  # Update every 5 seconds

except Exception as e:

print(f"[HEARTBEAT] Error: {e}")

time.sleep(5)

threading.Thread(target=heartbeat_loop, daemon=True).start()

print("[HEARTBEAT] Started heartbeat for backup system")

# Add this line in your main() function or at startup:

create_heartbeat()

3. Create Dual System Service

# Create service for backup system

sudo nano /etc/systemd/system/siseao-backup.service

[Unit]

Description=SISEAO Backup Controller

After=network.target

Requires=siseao-primary.service

[Service]

Type=simple

User=pi

WorkingDirectory=/home/pi/siseao_lite

ExecStart=/usr/bin/python3 /home/pi/siseao_lite/siseao_lite_backup.py

Restart=always

RestartSec=10

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

# Rename your primary service if needed

sudo systemctl stop siseao-lite.service

sudo systemctl disable siseao-lite.service

# Create primary service

sudo nano /etc/systemd/system/siseao-primary.service

[Unit]

Description=SISEAO Primary Controller

After=network.target

[Service]

Type=simple

User=pi

WorkingDirectory=/home/pi/siseao

ExecStart=/usr/bin/python3 /home/pi/siseao/rpi_appliance_controller.py

Restart=always

RestartSec=10

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

4. Enable Both Services

# Install psutil for process monitoring

pip3 install psutil

# Enable both services

sudo systemctl daemon-reload

sudo systemctl enable siseao-primary.service

sudo systemctl enable siseao-backup.service

# Start both services

sudo systemctl start siseao-primary.service

sudo systemctl start siseao-backup.service

# Check status

sudo systemctl status siseao-primary.service

sudo systemctl status siseao-backup.service

How It Works

Normal Operation (Primary Active)

- Primary Controller (rpi_appliance_controller.py):

  - Runs on Firebase integration

  - Controls GPIO pins directly

  - Creates heartbeat file every 5 seconds

  - Handles all device control

- Backup Controller (siseao_lite_backup.py):

  - Monitors GPIO pins (read-only mode)

  - Tracks energy usage locally

  - Provides monitoring web interface on port 5001

  - Detects primary controller status

Emergency Mode (Primary Offline)

- Backup Controller detects primary is down

- Switches to emergency control mode

- Allows direct GPIO control through web interface

- Shows warning messages

- All controls require confirmation

Access Points

- Primary System: http://[PI_IP]:5000 (Firebase-connected)

- Backup System: http://[PI_IP]:5001 (Local monitoring/emergency)

Benefits

✅ Redundancy: Backup control if primary fails ✅ Monitoring: Always track device states and energy ✅ No Conflicts: Backup monitors, primary controls ✅ Emergency Access: Local control when Firebase is down ✅ Dual Interfaces: Different access points for different needs

This setup gives you the best of both worlds - full Firebase integration with local backup capabilities!